10.1 Overview

10.2 Observatory Overheads

Table 10.1: Observatory Level Overheads.

Action	Overhead Time

Guide-star acquisition	First orbit, 6 minutes.

Guide-star reacquisitions	5 minutes at beginning of each new orbit for non–CVZ targets, 0 minutes for CVZ observing

New guide-star acquisition	6 minutes

following mosaic

dither >2 arcmin

Spacecraft re–pointing	For offsets between 10 and 90 arcseconds, 1 minute.

For offsets between 1 and 10 arcseconds, 0.5 minutes.

For offsets less than 1 arcsecond, 0.3 minutes.

Change UVIS quad filter, 1 minute.

10.3 Exposure Overheads

Table 10.2: WFC3 science exposure overhead times.

Action	Overhead Time (minutes)

Reconfiguration between UVIS & IR channels	1.0

during a single orbit

UVIS ACCUM

Single exposure or the first exposure in a	2.6

series of identical exposures.

Subsequent identical exposures in series	2.0

(within an orbit).

Serial buffer dump, during target visibility,	5.9

of one exposure (required if next UVIS exposure

time is less than 349 seconds)

IR MULTIACCUM

Single exposure or the first exposure in a	1.0

series of identical exposures.

Serial buffer dump, during target visibility,	5.9

of 15–sample exposure (required if next IR exposure

time is less than 349 seconds)

10.4.1 Example 1:  UVIS, 1 orbit, 1 filter

Table 10.3: Orbit calculation for example 1.

Action	Time	Explanation

(minutes)

Orbit 1

Initial guide-star acquisition	6.0	Needed at start of observation of a new target

WFC overhead for the first exposure	2.6	Includes filter change, camera set-up, and readout

First science exposure	20.0

UVIS overhead for the subsequent science	2.0	Includes readout

exposure in the series

The next science exposure in the series	20.0

Total time for science exposures	40.0

Total time used in the orbit visibility	51

10.4.2 Example 2:  UVIS, 1 orbit, 2 filters

Table 10.4: Orbit calculation for example 2.

Action	Time	Explanation

(minutes)

Orbit 1

Initial guide-star acquisition	6	Needed at start of observation of a new target

UVIS overhead for the first exposures	2  2.6 = 5.2	Includes filter change, camera set-up, and readout

in two series

UVIS overhead for subsequent	2  2.0 = 4	Includes readout

exposures in each of the two series

Additional overhead for all but the	5.9  2= 11.8	Needed to dump the buffer because the next

last exposures in the orbit		exposure is too short to

accommodate the dump time (270 sec < 349 sec).

Science exposures	4  4.5 = 18

Total time for science exposures	18

Total used time in the orbit	45

10.4.3 Example 3:  IR, 1 orbit, 2 filters

Table 10.5: Orbit calculation for example 3.

Action	Time	Explanation

(minutes)

Orbit 1

Initial guide-star acquisition	6	Needed at start of observation of a new target

Overhead for the 1st exposure	2  1.0 = 2	Includes filter changes, readouts, & camera set-up

Science exposures in F110W	11.7

Science exposures in F160W	23.4

Total time for science exposures	35.1

Total used time in the orbit	43

10.4.4 Example 4:  UVIS, dither, 1 filter

Table 10.6: Orbit calculation for example 4.

Action	Time	Explanation

(minutes)

Orbit 1

Initial guide-star acquisition	6	Needed at start of observation of a new target

UVIS overhead for the first exposures	2.6	Includes filter change, camera set-up, and readout

in the series

UVIS overhead for the subsequent	2.0	Includes readout

exposure

Spacecraft slew	0.5	To offset from the first to the second pointing

Two science exposures	2  20 = 40	Exposures at the first two pointings in the dither pattern

Total time for science exposures	40

Total used time in the orbit	51

Orbit 2

Guide-star re-acquisition	5	Needed at start of a new orbit to observe the same target

UVIS overhead for the remaining	2  2.0 = 4	Includes readout

Two exposures

Spacecraft slews	2  0.5 = 1	To offset to the third and fourth pointings

Two science exposures	2  20 = 40	Exposures at the final two pointings in the dither pattern

Total time for science exposures	40

Total time used in the orbit	50

10.4.5 Example 5:  IR, 1 orbit, grism

Table 10.7: Orbit calculation for example 5.

Action	Time	Explanation

(minutes)

Initial guide-star acquisition	6	Needed at start of observation of a new target

IR overhead for the first	1.0	Includes filter change, camera set-up, and readout

science exposure

Undispersed imaging	2.4	Needed to co-locate the targets and their spectra in the FOV

IR overhead for the grism	1.0	Includes readout

exposure

Grism exposure	46.7

Total science exposure time	49.1

Total used time in the orbit	57.1